Band Preparation for in-gel digestion

- Cut gel bands out of gel and cut into 1-2 mm cubes on a glass surface (cleaned well with 70% ethanol) using a clean scalpel or razor. Cut carefully around the band or region of interest, minimizing the inclusion of excess gel in the sample. Work quickly to prevent gel from drying out. Hydrate with DI water if needed. Clean tool with 70% ethanol between samples if processing multiple bands.

- Transfer gel pieces from each band or region into individual Eppendorf tubes containing enough DI water to cover bands completely. At 4C until ready to process, but no more than a couple of days.